# School Site Selection Using Spatial Analysis

Project: Spatial Data Analysis - Homework 1
Authors: Ameer Saleh & Bara Mhana
Date: December 1, 2025

## Executive Summary

This project identifies suitable locations for new school construction using PostGIS spatial analysis and QGIS visualization. From 140 initial land parcels, 7 suitable sites were identified totaling 52,779.20 m² through systematic application of four selection criteria.

Key Results:

- 7 suitable parcels identified (5% success rate)

- Average parcel size: 7,539.89 m²

- 85.7% of sites directly adjacent to roads

- Mix of un-used (71.4%) and agricultural lands (28.6%)

## 1. Introduction

### 1.1 Objectives

Identify suitable land parcels for school construction using multi-criteria spatial analysis in PostGIS with the following requirements:

- Land use types: Un-Used, Agricultural, or Commercial

- Minimum area: 5,000 m²

- No existing buildings on parcels

- Within 25m of nearest road

### 1.2 Tools and Technologies

- PostgreSQL + PostGIS - Spatial database and analysis

- QGIS 3.16+ - Geographic visualization

- SQL - Query development

## 2. Data and Study Area

### 2.1 Database Schema

Landuse Table (Primary dataset)

- idd - Primary key

- type - Land use classification

- area - Parcel area (m²)

- geom - Polygon geometry

Buildings Table

- idd - Primary key

- geom - Building footprints

Roads Table

- idd - Primary key

- name - Road name

- geom - Road centerlines

## 3. Methodology

### 3.1 Analysis Workflow

Initial Landuse (140 parcels)

↓

① Land Use Type Filter → 140 parcels

↓

② Area ≥ 5000 m² → 23 parcels

↓

③ No Buildings → 17 parcels

↓

④ Road Proximity ≤ 25m → 7 parcels (FINAL)

### 3.2 Sequential Filtering Approach

Each criterion progressively narrows the candidate set, ensuring all requirements are met while optimizing query performance.

## 4. Selection Criteria

### Criterion 1: Land Use Type

Requirement: Un-Used, Agricultural Areas, or Commercial Lands

WHERE type IN ('Un-Used', 'Agricultural Areas', 'Commercial Lands')

Result: 140 parcels retained

### Criterion 2: Minimum Area

Requirement: Area ≥ 5,000 m²

AND ST_Area(geom) >= 5000

Result: 23 parcels (83.6% eliminated)

### Criterion 3: No Buildings

Requirement: No building footprints intersecting parcels

AND NOT EXISTS ( SELECT 1 FROM buildings b WHERE ST_Intersects(l.geom, b.geom) )

Result: 17 parcels (26.1% eliminated)

### Criterion 4: Road Proximity

Requirement: Within 25m of nearest road

AND EXISTS ( SELECT 1 FROM roads r WHERE ST_DWithin(l.geom, r.geom, 25) )

Result: 7 parcels (58.8% eliminated)

## 5. Technical Implementation

### 5.1 Complete SQL Query

-- Final query applying all criteria CREATE TABLE final_school_locations AS SELECT l.idd, l.type, l.owner, l.area, l.geom, ST_Area(l.geom) as calculated_area, (SELECT MIN(ST_Distance(l.geom, r.geom)) FROM roads r) as distance_to_nearest_road FROM landuse l WHERE -- Criterion 1: Land use type l.type IN ('Un-Used', 'Agricultural Areas', 'Commercial Lands') -- Criterion 2: Minimum area AND ST_Area(l.geom) >= 5000 -- Criterion 3: No buildings AND NOT EXISTS ( SELECT 1 FROM buildings b WHERE ST_Intersects(l.geom, b.geom) ) -- Criterion 4: Road proximity AND EXISTS ( SELECT 1 FROM roads r WHERE ST_DWithin(l.geom, r.geom, 25) ) ORDER BY ST_Area(l.geom) DESC; -- Add primary key and spatial index ALTER TABLE final_school_locations ADD PRIMARY KEY (idd); CREATE INDEX idx_final_sites_geom ON final_school_locations USING GIST(geom);

## 6. Results and Analysis

### 6.1 Filtering Progression

| Step | View Name | Count | Criteria Applied |
| --- | --- | --- | --- |
| 1 | suitable_landuse_types | 140 | TYPE IN (Un-Used, Agricultural, Commercial) |
| 2 | suitable_area_parcels | 23 | ST_Area(geom) ≥ 5,000 m² |
| 3 | parcels_without_buildings | 17 | NOT ST_Intersects with buildings |
| 4 | final_school_locations | 7 | ST_DWithin roads 25m |

Overall Reduction: 95.0% (140 → 7 parcels)

### 6.2 Final School Sites

| ID | Land Use Type | Area (m²) | Distance to Road (m) |
| --- | --- | --- | --- |
| 20 | Agricultural Areas | 9,935.86 | 0.00 |
| 131 | Un-Used | 8,247.02 | 0.00 |
| 215 | Un-Used | 7,981.13 | 0.00 |
| 8 | Un-Used | 7,283.46 | 0.00 |
| 145 | Un-Used | 7,214.55 | 0.00 |
| 11 | Un-Used | 6,255.88 | 14.22 |
| 17 | Agricultural Areas | 5,861.30 | 0.00 |

### 6.3 Summary Statistics

Area Analysis:

- Total Area: 52,779.20 m²

- Mean: 7,539.89 m²

- Minimum: 5,861.30 m²

- Maximum: 9,935.86 m²

Distance to Roads:

- Mean: 2.03 m

- Sites adjacent to roads: 6 (85.7%)

- Maximum distance: 14.22 m

Land Use Distribution:

| Land Use Type | Count | Total Area (m²) | Percentage |
| --- | --- | --- | --- |
| Un-Used | 5 | 36,982.04 | 71.4% |
| Agricultural Areas | 2 | 15,797.16 | 28.6% |
| Total | 7 | 52,779.20 | 100% |

## 7. QGIS Visualization

### 7.1 Loading Data in QGIS

Method 1: Direct PostGIS Connection

- Layer → Add Layer → Add PostGIS Layers

- Create connection to database

- Select final_school_locations table

- Add to map

Method 2: DB Manager

- Database → DB Manager → PostGIS

- SQL Window → Execute query

- Check "Load as new layer"

### 7.2 Map Styling

Final School Sites:

- Fill: Green (#4CAF50), 60% opacity

- Outline: Dark Green (#2E7D32), 1.5pt

- Labels: Site ID and area

Supporting Layers:

- Roads: Gray lines (1.5pt)

- Buildings: Red outlines (0.5pt)

- 25m Road Buffer: Blue dashed (40% opacity)

### 7.3 Map Layout Elements

- Title: "Suitable School Sites Analysis"

- Legend with all layers

- Scale bar and north arrow

- Data sources and projection info

- Results summary table

## 8. Conclusions and Recommendations

### 8.1 Summary

Successfully identified 7 suitable parcels totaling 52,779 m² through rigorous spatial analysis. All sites meet the four selection criteria and are development-ready with excellent road access.

### 8.2 Immediate Next Steps

- Site Verification - Field visits to all 7 sites

- Environmental Assessment - Phase I environmental studies

- Cost Analysis - Land appraisals and development estimates

- Stakeholder Engagement - Community consultation and owner contact

### 8.3 Future Enhancements

Additional Analysis:

- Multi-criteria decision analysis (MCDA) with weighted criteria

- Cost-benefit analysis integration

- Student population density mapping

- Environmental constraints (slopes, flood zones)

- Traffic and accessibility modeling

Data Improvements:

- Property values and ownership details

- Existing school locations and capacity

- Demographic and enrollment projections

- Utility infrastructure availability

Report prepared by: Ameer Saleh & Bara Mhana
Course: Spatial Data Analysis
Assignment: Homework 1 - Site Selection using PostGIS
Date: December 1, 2025

END OF REPORT