Tree Cutting Priority Analysis for Fire Creek

Project: Spatial Data Analysis - Homework 2
Authors: Ameer Saleh & Bara Mhana
 Date: December 12, 2025

Executive Summary

This project calculates and prioritizes tree cutting zones for wildfire risk mitigation in the Fire Creek area using Python-based spatial analysis with GeoPandas and matplotlib visualization.

Key Results:

- Zones Analyzed: 80 cutting grid zones

- Method: Multi-criteria decision analysis (MCDA) using five weighted factors

- Normalization: Scores scaled 0-100

- Priority Classes: High, Medium, Low

Key Metrics:

- Mean Priority Score: 29.37

- Maximum Priority Score: 60.73 (Grid 163)

- Standard Deviation: 13.71

Distribution:

- Critical Priority: 0 zones (0.0%)

- High Priority: 5 zones (6.3%)

- Medium Priority: 43 zones (53.8%)

- Low Priority: 32 zones (40.0%)

Outputs: Complete visualization with 3 analytical charts and spatial data files

1. Introduction

1.1 Objectives

The primary objective is to calculate tree cutting priority for each zone using Multi-Criteria Decision Analysis (MCDA) in Python/GeoPandas to support wildfire risk mitigation efforts in the Fire Creek area.

Analysis Factors & Weights:

- Tree Mortality: 30%

- Community Features: 15%

- Egress Routes: 20%

- Populated Areas: 20%

- Electric Utilities: 15%

1.2 Tools and Technologies

- Python 3.x - Core programming language

- GeoPandas - Spatial analysis and operations

- Pandas / NumPy - Data processing and mathematical operations

- Matplotlib - Charts and visualization generation

- QGIS 3.16+ - Geographic visualization and mapping

- Shapefile / GeoPackage - Spatial data formats

2. Data and Study Area

2.1 Study Area

Location: Fire Creek Area
 Spatial Reference System: EPSG:26711
 Analysis Units: 80 grid zones

2.2 Input Data Schema

| Dataset | Features | Description |
| --- | --- | --- |
| CuttingGrids | 80 | Grid polygons defining cutting zones |
| SBNFMortality | 12 | Tree mortality polygons with total mortality values |
| CommunityFeature | 8 | Locations of critical community infrastructure |
| EgressRoutes | 6 | Emergency evacuation route lines |
| PopulatedAreas | 6 | Residential / populated area polygons |
| Electric Utilities | 5 layers | Transmission, SubTransmission, Distribution Circuits, Substations, PoleTopSubs |

3. Methodology

3.1 Analysis Workflow

Initial Dataset (80 Grid Zones + 5 Risk Factors)
↓
┌────────────────────────────────────────────┐
│ STEP 1: Load & Align CRS → EPSG:26711     │
└────────────────┬───────────────────────────┘
                 ↓
┌────────────────────────────────────────────┐
│ STEP 2: Calculate Mortality Score         │
│ → Spatial intersection with mortality data│
│ → Normalize 0-100                          │
└────────────────┬───────────────────────────┘
                 ↓
┌────────────────────────────────────────────┐
│ STEP 3: Calculate Community Score         │
│ → Count community features per zone        │
│ → Normalize 0-100                          │
└────────────────┬───────────────────────────┘
                 ↓
┌────────────────────────────────────────────┐
│ STEP 4: Calculate Egress Score            │
│ → Measure route coverage per zone          │
│ → Normalize 0-100                          │
└────────────────┬───────────────────────────┘
                 ↓
┌────────────────────────────────────────────┐
│ STEP 5: Calculate Populated Score         │
│ → Assess residential area exposure         │
│ → Normalize 0-100                          │
└────────────────┬───────────────────────────┘
                 ↓
┌────────────────────────────────────────────┐
│ STEP 6: Calculate Utility Score           │
│ → Evaluate 5 utility layer exposures       │
│ → Normalize 0-100                          │
└────────────────┬───────────────────────────┘
                 ↓
┌────────────────────────────────────────────┐
│ STEP 7: Apply Weighted MCDA               │
│ → Priority Score = Σ(weight × score)      │
│ → Final score range: 0-100                │
└────────────────┬───────────────────────────┘
                 ↓
┌────────────────────────────────────────────┐
│ STEP 8: Classify Priority                 │
│ → Low / Medium / High / Critical           │
└────────────────┬───────────────────────────┘
                 ↓
┌────────────────────────────────────────────┐
│ STEP 9: Generate Visualizations           │
│ → Bar Chart, Pie Chart, Top 10 Chart      │
└────────────────────────────────────────────┘

3.2 MCDA Approach

The analysis employs a weighted Multi-Criteria Decision Analysis approach where:

- Each factor is independently calculated and normalized to a 0-100 scale

- Weighted summation produces the final priority score

- Higher scores indicate higher priority for tree cutting operations

3.3 Normalization Formula

For each criterion, raw scores are normalized using:

Normalized Score = (Raw Score / Maximum Raw Score) × 100

This ensures all factors contribute proportionally to the final priority score regardless of their original measurement units.

4. Scoring Criteria

4.1 Overview of Criteria

| Criterion | Weigh | Description |
| --- | --- | --- |
| Tree Mortality | 30% | Spatial intersection with mortality polygons |
| Community Features | 15% | Presence of critical community infrastructure |
| Egress Routes | 20% | Coverage of evacuation routes |
| Populated Areas | 20% | Exposure of residential areas |
| Electric Utilities | 15% | Exposure of utility infrastructure |

4.2 Criterion 1: Tree Mortality (30%)

Rationale: Dead and dying trees pose the highest direct fire risk, serving as fuel sources that can accelerate wildfire spread and intensity.

Implementation:

- Spatial intersection between cutting grids and mortality polygons

- Sum of total mortality values within each zone

- Higher mortality values indicate greater immediate risk

Impact: Primary driver of priority scores, reflecting direct fire hazard from dead vegetation.

4.3 Criterion 2: Community Features (15%)

Rationale: Protection of critical infrastructure including schools, fire stations, hospitals, and community centers is essential for public safety and emergency response capability.

Implementation:

- Count of community features within or near each zone

- Binary scoring for presence/absence of critical facilities

- Proximity weighting for features near zone boundaries

Impact: Ensures protection of community assets vital for evacuation and emergency services.

4.4 Criterion 3: Egress Routes (20%)

Rationale: Clear evacuation routes are life-critical during wildfire events. Tree removal along escape corridors prevents route blockage and ensures safe evacuation.

Implementation:

- Calculate length of egress routes passing through each zone

- Higher coverage indicates greater importance for evacuation safety

- Line intersection analysis with grid zones

Impact: Prioritizes zones that, if left unmanaged, could block evacuation pathways.

4.5 Criterion 4: Populated Areas (20%)

Rationale: Residential areas face direct threat from wildfire. Tree cutting creates defensible space around homes and reduces ember ignition risks.

Implementation:

- Measure overlap between populated area polygons and cutting zones

- Area-based scoring reflecting population exposure

- Higher scores for zones with greater residential coverage

Impact: Protects human life and property by creating fire breaks near homes.

4.6 Criterion 5: Electric Utilities (15%)

Rationale: Utility infrastructure poses dual risk: power lines can spark fires, and fires can damage critical electrical infrastructure causing widespread outages.

Electric Utility Scoring Details:

| Utility Type | Weigh | Method |
| --- | --- | --- |
| Transmission Lines | 3.0 | Length × weight × priority |
| SubTransmission Lines | 2.5 | Length × priority |
| Distribution Circuits | 2.0 | Length within zone |
| Substations | 3.0 | Fixed score × priority |
| Pole Top Substations | 2.0 | Count × fixed score |

Implementation:

- Multi-layer analysis across 5 utility datasets

- Weighted scoring based on voltage level and criticality

- Length-based calculations for linear features (power lines)

- Count-based calculations for point features (substations)

Impact: Reduces fire ignition risk from electrical equipment and protects power grid reliability.

5. Priority Classification

5.1 Classification System

| Priority Class | Score Range | Action Required | Description |
| --- | --- | --- | --- |
| Critical | 75-100 | Immediate action | Emergency response within days |
| High | 50-74 | Near-term action | Schedule within current season |
| Medium | 25-49 | Scheduled maintenance | Address in annual work plan |
| Low | 0-24 | Routine monitoring | Monitor and reassess periodically |

5.2 Weighted Priority Formula

The final priority score for each zone is calculated as:

priority_score = (0.3 × mortality_score) + (0.15 × community_score) + (0.2 × egress_score) + (0.2 × populated_score) + (0.15 × utility_score)

Where each component score is normalized to 0-100 scale before weighting.

6. Results and Analysis

6.1 Priority Distribution

Analysis Results:

| Class | Coun | Percentage | Cumulative |
| --- | --- | --- | --- |
| Critical | 0 | 0.0% | 0.0% |
| High | 5 | 6.3% | 6.3% |
| Medium | 43 | 53.8% | 60.0% |
| Low | 32 | 40.0% | 100.0% |
| Total | 80 | 100% | - |

Key Insights:

- No zones reached critical threshold (75+), indicating manageable overall risk

- Only 5 zones (6.3%) require near-term action

- Majority (53.8%) fall into medium priority requiring scheduled maintenance

- 40% are low priority suitable for routine monitoring

6.2 Score Statistics

Statistical Summary:

| Metric | Value | Interpretation |
| --- | --- | --- |
| Mean Priority Score | 29.37 | Average zone falls in medium-low range |
| Maximum Priority Score | 60.73 | Highest risk zone (Grid 163) |
| Minimum Priority Score | 8.16 | Lowest risk zone |
| Standard Deviation | 13.71 | Moderate variability across zones |
| Median | 26.45 | Central tendency near mean |

Distribution Characteristics:

- Right-skewed distribution with concentration in 20-40 score range

- No extreme outliers above 65 points

- Relatively consistent risk levels across most zones

6.3 High Priority Zones

Top 5 Zones Requiring Near-Term Action:

| Rank | Grid ID | Score | Class | Key Risk Factors |
| --- | --- | --- | --- | --- |
| 1 | 163 | 60.73 | High | High mortality, egress route coverage |
| 2 | 158 | 55.51 | High | Community features, utility exposure |
| 3 | 130 | 54.48 | High | Populated area proximity, mortality |
| 4 | 157 | 51.84 | High | Multiple utility layers, egress routes |
| 5 | 162 | 50.74 | High | Balanced risk across all factors |

Operational Recommendations:

- Grid 163 should be addressed first (highest score)

- All five zones should be scheduled within current fire season

- Field verification recommended for final work planning

6.4 Top 10 Priority Zones

Extended Priority List:

| Rank | Grid ID | Priority Score | Class | Notes |
| --- | --- | --- | --- | --- |
| 1 | 163 | 60.73 | High | Top priority |
| 2 | 158 | 55.51 | High | Secondary priority |
| 3 | 130 | 54.48 | High | Community protection focus |
| 4 | 157 | 51.84 | High | Utility corridor |
| 5 | 162 | 50.74 | High | Egress route critical |
| 6 | 168 | 48.69 | Medium | Near high threshold |
| 7 | 154 | 48.57 | Medium | Consider for early action |
| 8 | 172 | 47.73 | Medium | Moderate risk balanced |
| 9 | 124 | 47.21 | Medium | Monitor for escalation |
| 10 | 113 | 45.70 | Medium | Standard maintenance |

Planning Considerations:

- Zones 6-10 are medium priority but near high threshold

- Consider advancing zones 6-7 if resources permit

- Group adjacent zones for operational efficiency

6.5 Spatial Analysis

Spatial Patterns Observed:

- High priority zones clustered in central-eastern portion of study area

- Spatial correlation with populated areas and utility corridors

- Linear patterns following egress routes and transmission lines

- Low priority zones concentrated in peripheral areas with less infrastructure

7. Visualization and Outputs

7.1 Chart Types

Three complementary visualizations were generated to support decision-making:

- Bar Chart - Grid Zones by Priority Class

  - Shows count of zones in each priority category

  - Highlights the 43 medium-priority zones requiring scheduled attention

  - Confirms no critical zones requiring emergency response

- Pie Chart - Priority Class Distribution

  - Percentage breakdown: Low (40%), Medium (53.8%), High (6.2%)

  - Visual representation of overall risk profile

  - Demonstrates manageable risk distribution

- Horizontal Bar Chart - Top 10 Highest Priority Zones

  - Individual zone scores with Grid IDs

  - Color-coded by classification (orange for high, yellow for medium)

  - Facilitates crew assignment and work scheduling

7.2 Color Scheme

Consistent color coding used across all visualizations:

| Priority Class | Color | Application |
| --- | --- | --- |
| Critical | Red | Emergency zones (none in current analysis) |
| High | Orange | Near-term action (5 zones) |
| Medium | Yellow | Scheduled maintenance (43 zones) |
| Low | Green | Routine monitoring (32 zones) |

Chart Output: TreeCuttingPriority_Charts.png (300 DPI, publication quality)

7.3 QGIS Visualization

Loading Priority Results in QGIS

Method 1: Add Shapefile

- Layer → Add Layer → Add Vector Layer

- Browse to TreeCuttingPriority_[timestamp].shp

- Click Add to load into map canvas

Method 2: Add GeoPackage

- Layer → Add Layer → Add Vector Layer

- Select TreeCuttingPriority_[timestamp].gpkg

- Choose layer from package if multiple layers present

Styling by Priority Class

Categorized Symbology:

- Right-click layer → Properties → Symbology

- Select "Categorized" renderer

- Field: prior_cls (priority class)

- Click "Classify" to auto-generate classes

- Assign colors:

  - Critical → Red, opacity 70%

  - High → Orange, opacity 70%

  - Medium → Yellow, opacity 60%

  - Low → Green, opacity 60%

- Adjust outline: Black, 0.5pt width

Alternative: Graduated Colors

- Field: priority_s (priority score)

- Mode: Pretty Breaks, 4 classes

- Color Ramp: RdYlGn (inverted - red high, green low)

- Manual adjustments for breakpoints: 0-25, 25-50, 50-75, 75-100

8. Conclusions and Recommendations

Summary

This GeoPandas-based spatial analysis successfully evaluated all 80 Fire Creek cutting zones using multi-criteria decision analysis. The systematic approach:

✓ Applied five weighted risk factors using industry-standard wildfire mitigation criteria

✓ Normalized all scores to comparable 0-100 scale

✓ Identified 5 high-priority zones requiring near-term action

✓ Classified 43 zones for scheduled maintenance programs

✓ Produced reproducible, data-driven results through documented Python workflows

✓ Generated professional visualizations for stakeholder communication

Strengths of Analysis:

- Objectivity - Mathematical criteria eliminate subjective bias in prioritization

- Transparency - Weighted formula clearly documented and auditable

- Spatial Intelligence - Geographic relationships drive meaningful risk assessment

- Actionable Results - Clear priority classes support operational planning

- Reproducibility - Python code enables reanalysis with updated data

Report Prepared by: Ameer Saleh & Bara Mhana
Course: Spatial Data Analysis
Assignment: Homework 3 - Tree Cutting Priority Analysis
Date: December 12, 2025

END OF REPORT